# Q50 Site/ street furniture/ equipment

To be read with Preliminaries/ General conditions.

SITE AND STREET FURNITURE

210	CYCLE STANDS

•	 Manufacturer: Contractor's choice.

-	Product reference: Sheffield stands.

•	 Material: Steel.

-	Finish: hot dip galvanised.

-	Colour: None.

•	 Number of stands: 5.

•	 Accessories: None.

•	 Method of fixing: In accordance with manufacturers recommendations.

220A	BENCHES - NURSERY PLAYGROUND

•	 Manufacturer: Bespoke - refer to detail drawings.

Workmanship: in accordance with Section Z10

•	 Material: Douglas fir or approved alternative.

Size: refer to drawings.

•	 Accessories/ Special requirements: lockable storage boxes below seating adjacent to

new extension: curved bench around tree; climbing plant structure bove seat by timber

shed.

221	SHED - NURSERY PLAYGROUND

•	 Manufacturer: Bespoke - refer to detail drawings.

Workmanship: in accordance with Section Z10

Material: Douglas fir or approved alternative.

Size: refer to drawings.

•	 Accessories/ Special requirements: include all ironmongery.

INSTALLATION

510	CONCRETE FOUNDATIONS GENERALLY

•	 Standard: To 8500-2.

•	 Mix: Designated concrete not less than GEN 1 or standard prescribed concrete not

less than ST2.

•	 Admixtures: Do not use.

•	 Foundation holes: Neat vertical sides.

•	 Depth of foundations, bedding, haunching: Appropriate to provide adequate support

and to receive overlying soft landscape or paving finishes.