L30	STAIRS/ LADDERS/ WALKWAYS/ HANDRAILS/ BALUSTRADES

To be read with Preliminaries/ General conditions.

PRELIMINARY INFORMATION/ REQUIREMENTS

L30/110	DESIGN

-	Design standard: To be in accordance with BS 5395 where applicable: Stairs, Walkways and Balustrades.

-	Completion of design: Finalise details to meet structural and safety requirements of BS 5395.

-	Type of activity/ occupancy category to BS 6399-1: C

L30/130	SITE DIMENSIONS

-	Procedure: Before starting work on designated items take site dimensions, record on shop drawings and use to ensure accurate fabrication.

COMPONENTS

570A	PURPOSE MADE HANDRAILS TO 2NO. RAMPS AND 2NO. STAIR FLIGHTS

•	 Component material, grade and finish as delivered:

- Handrails: CHS low carbon (mild) steel, galvanized to BS EN ISO 1461 and

coated with Plascoat PPA 571.

-	Brackets and legs: Low carbon steel - galvanized and painted .

•	 Workmanship:

-	Joinery: Not applicable.

-	Metalwork: To section Z11.

•	 Other requirements: Bolt assemblies to be galvanized..

•	 Fixing: Refer to drawings

INSTALLATION

L30/620	PRIMING/ SEALING/ PAINTING

-	Surfaces inaccessible after assembly/installation: Before fixing components, apply full protective/decorative treatment/coating system.

L30/630	CORROSION PROTECTION OF DISSIMILAR MATERIALS

-	Components/ substrates/ fasteners of dissimilar materials: Isolate using washers/ sleeves or other suitable means to separate materials to avoid corrosion and/ or staining.

L30/640	INSTALLATION GENERALLY

-	Fasteners and methods of fixing: To section Z20.

-	Structural members: Do not modify, cut, notch or make holes in structural members, except as indicated on drawings.

-	Temporary support: Do not use stairs, walkways or balustrades as temporary support or strutting for other work.