M51	EDGE FIXED CARPETING

To be read with Preliminaries/ General conditions.

TYPES OF CARPETING

M51/110	CARPETING

-	Location: – see drawings

-	Base: Screed

-	Preparation: As clause nos. 340 & 360.

-	Fabricated underlay: Not required

-	Carpet:

-	Manufacturer: Heckmondwike

Product reference: Supacord

-	Colour/ pattern: To be approved

-	Width: To be approved

-	Method of seaming: As manufacturer’s recommendations

-	Method of fixing: Carpet gripper.

-	Methods of fixing at openings/ free edges: As clause no. 510

GENERAL/ PREPARATION

M51/210	WORKMANSHIP GENERALLY

-	Finished carpeting: Tightly seamed, accurately fitted, neatly and securely fixed, smooth and evenly tensioned.

M51/250	LAYOUT

-	Setting out: Before placing order, agree seam locations and pattern.

M51/290	CONDITIONING

-	Requirements: As recommended by manufacturer.

M51/310	COMMENCEMENT

-	Required condition of works prior to laying carpeting:

-	Building is weathertight and well dried out.

-	Wet trades have finished work.

-	Paintwork is finished and dry.

-	Floor service outlets, duct covers and other fixtures around which carpet is to be cut are fixed.

-	Notification: Submit not less than 48 hours before commencing laying.

M51/320	ENVIRONMENT

-	Temperature and humidity: Before, during and after laying, maintain approximately at levels which will prevail after building is occupied.

M51/330	BASES

-	Suitability of bases and conditions within any area: Commencement of laying carpeting will be taken as acceptance of suitability.

M51/340	NEW WET LAID BASES

-	Base drying aids: Not used for at least four days prior to moisture content testing.

-	Base moisture content test: Carry out in accordance with BS 5325, Annexe A.

-	Locations for readings: In all corners, along edges, and at various points over area being tested.

-	Commencement of laying carpeting: Not until all readings show 75% relative humidity or less.

M51/360	EXISTING FLOORING TO BE OVERLAID

-	Substrate: Make good by local rebedding, sanding or applying smoothing underlayment compound to give a secure, smooth, even surface. Allow to dry before laying carpeting.

LAYING CARPETING

M51/470	LAYING CARPET GENERALLY

-	Appearance of laid carpet: Pieces of the same carpet type capable of being seen together to be of consistent appearance with pile lying in the same direction.

-	Carpet perimeter: Accurately and closely fitted leaving no gaps with edges turned down and secured to grippers.

-	Carpet tension: Even and such that carpet is flat and will not ruck, ripple or become slack.

-	Doorways and recesses: Cut carpet in. Do not piece in without prior approval.

M51/480	POWER STRETCHING

-	General: Power stretch carpets with any one dimension equal to or above 5m.

M51/490	DOORWAYS

-	Carpet joint: On centre line of door leaf.

M51/510	EDGINGS/ COVER STRIPS

-	Manufacturer: Contractor’s choice

-	Product reference: Contractor’s choice subject to approval

-	Material/ finish: aluminium

-	Fixing: Secure (using matching fasteners where exposed to view) with edge of carpet firmly gripped.

M51/570	COMPLETION

-	Debris: Remove stay tacks and cut away partly loose warp and face yarns.

-	Surface irregularities and tension: Check and make necessary tension adjustments.

M51/580	WASTE

-	Spare covering material: Retain suitable material for patching. On completion submit pieces for selection. Hand over selected pieces to Employer.