H32	PLASTICS PROFILED SHEET CLADDING/ COVERING

To be read with Preliminaries/ General conditions.

H32/120	PLASTICS COVERING TO ROOF OVER BIKE STORE AND NURSERY PLAYGROUND

-	Support structure: See Structural Engineer’s details .

-	Pitch: Minimum recommended roof pitch 5˚.

-	Spanning form: Single.

-	System type: Single skin factory assembled.

-	Manufacturer: Filon Products Ltd. Tel Nº 01543 687 300.

-	Product reference: 10” x 3” Profile (SHOE  001B), 2.5mm thick Supasafe .

-	Fragility class to ACR(M)001: A.

-	Accessories: Profile fillers as clasue H32/220.

-	Primary sheet fasteners type: As recommended by Manufacturer.

-	Fastener profile location: As recommended by Manufacturer.

-	Number of fasteners per sheet width: As recommended by Manufacturer.

Eaves and end laps: As recommended by Manufacturer.

Intermediate supports: As recommended by Manufacturer.

-	End laps size (minimum): As recommended by Manufacturer.

-	Sealing: As recommended by Manufacturer.

-	Side laps:

-	Sealing: As recommended by Manufacturer

-	Stitching: As recommended by Manufacturer.

-	Maximum unsupported overhang: 250mm .

-	Special features: None.

H32/191	WATER PENETRATION

-	Requirement: Under site exposure conditions moisture must not penetrate onto internal surfaces, or into cavities not designed to be wetted.

H32/210	PAINTING STRUCTURE

-	Sequence: Paint outer surface of supporting structure before fixing cladding/ covering (bike store only).

H32/213	FASTENERS

-	Unspecified fasteners: Recommended for the purpose by the cladding/ covering manufacturer.

H32/217	ACCESSORIES

-	Unspecified accessories: Recommended for the purpose by the cladding/ covering manufacturer.

H32/220	PROFILE FILLERS

-	Types: Supplied by cladding manufacturer accurately matching sheet profile.

-	Material/ colour: As recommended by Manufacturer.

-	Locations: As shown on drawings, on the line of or above, fasteners and wherever necessary to close off profile cavities from the outside and inside of the building.

-	Fit: Tight with no gaps.

-	Profile fillers in sealed laps: Bed in sealant on top and bottom surfaces.

H32/230	FIXING SHEETS GENERALLY

-	Cut edges: Clean true lines.

-	Sheet orientation: Exposed joints of side laps away from prevailing wind unless shown otherwise on drawings.

-	Fastener hole location: At regular intervals in straight lines and not less than 50 mm from edges of sheets and fittings.

-	Crown fixing: For sheets with a profile depth greater than 20 mm support crowns at primary fixings with profile fillers.

-	Debris: No dust or foreign matter to be present within construction.

-	Fasteners torque: Sufficient to correctly compress washer.

-	On completion: Fixings to be watertight and sheets secure with no buckling or distortion.

H32/260	ACCOMMODATION OF THERMAL MOVEMENT

-	PVC-U and polycarbonate sheets: Use oversize holes for primary fixings in accordance with sheet manufacturer’s recommendations.

H32/320	ABUTMENTS

-	Junctions with flashings: Watertight and neatly dressed.

H32/330	SEALING LAPS

-	Sealant tape: Types recommended for the purpose by sheet manufacturer.

-	End laps: Position sealant tape in straight, unbroken lines below fixing positions, parallel to and slightly back from edge of sheet.

-	Side laps: Position sealant tape outside the line of fasteners. Where a second tape is specified, position on the other side of the fasteners.

-	Laps between plastics sheets and other materials: Use sealant tape specified for other materials.

-	Seal quality: Effective, continuous and not overcompressed.

H32/340	WARNING NOTICES

-	Fixing locations of signs: to be agreed .

-	Manufacturer: Contractor’s choice .

-	Material: Shock and weather resistant .

-	Signs description: As BS 5499-5 code 8.C.0072 with supplementary text sign, lettering 'DANGER Fragile roof'.

Mandatory sign as BS 5499-5 code 11.A.0103 with supplementary text sign, lettering 'Use crawling boards'.