# Q28 Topsoil and growing media

To be read with Preliminaries/ General conditions.

- PORTED TOPSOIL TO BS 3882 IN RAISED BEDS IN NURSERY PLAYGROUND

•	 Quantity: Provide as necessary to make up any deficiency of topsoil existing on site

and to complete the work.

•	 Standard: To BS 3882.

•	 Classification: Multipurpose.

•	 Source: Contractor's choice.

•	 Submit: Declaration of analysis in accordance with BS 3882, Annex E.

•	 Additional analyses: Not required.

670	CONTAMINATION

•	 General: Do not use topsoil contaminated with subsoil, rubbish or other materials that

are:

-	Corrosive, explosive or flammable.

-	Hazardous to human or animal life.

-	Detrimental to healthy plant growth.

•	 Subsoil: In areas to receive topsoil, do not use subsoil contaminated with the above

materials.

•	 Give notice: If any evidence or symptoms of soil contamination are discovered on the

site, or in topsoil to be imported.

690	HANDLING TOPSOIL

•	 Aggressive weeds: Give notice and obtain instructions before moving topsoil.

•	 Plant: Select and use plant to minimize disturbance, trafficking and compaction.

•	 Contamination: Do not mix topsoil with:

-	Subsoil, stone, hardcore, rubbish or material from demolition work.

-	Other grades of topsoil.

•	 Multiple handling: Keep to a minimum. Use topsoil immediately after stripping.

•	 Wet conditions: Handle topsoil in the driest condition possible. Do not handle during

or after heavy rainfall or when it is wetter than the plastic limit less 3%, to BS 1377-2.