P20	UNFRAMED ISOLATED TRIMS/ SKIRTINGS/ SUNDRY ITEMS

To be read with Preliminaries/ General conditions.

P20/110	SOFTWOOD FOR SKIRTING GENERALLY

-	Quality of wood and fixing: To BS 1186-3.

-	Species: Contractors choice.

-	Class: CSH and 1.

-	Moisture content at time of fixing: 6% to 10%.

-	Preservative treatment: none.

- Profile: Pencil rounded

- Finished size : 19 x 44mm to existing building generally with tiled dado, 19 x 94mm elsewhere (except where specified for integral coved tiled).

-	Finish as delivered: prepared + primed.

- Fixing: Lost head nail.

P20/111	SOFTWOOD FOR MISCELLANEOUS TRIMS

-	Quality of wood and fixing: To BS 1186-3.

-	Species: Contractors choice.

-	Class: CSH and 1.

-	Moisture content at time of fixing: 6% to 10%.

-	Preservative treatment: none.

- Profile: Pencil rounded

-	Finish as delivered: prepared and primed

- Fixing: Lost head nail.

P20/112	SOFTWOOD FOR ARHITRAVES GENERALLY, INCLUDING AT ACOUSTIC ABSORBERS, EXCEPT AS CLAUSE 113

-	Quality of wood and fixing: To BS 1186-3.

-	Species: Contractors choice.

-	Class: CSH and 1.

-	Moisture content at time of fixing: 6% to 10%.

-	Preservative treatment: none.

- Profile: PSE

- -Finished size: 44 x 19mm at acoustic absorbers, 69 x 19 elsewhere.

-	Finish as delivered: prepared and primed.

-	Fixing: Lost head nail.

P20/113	SOFTWOOD FOR ARHITRAVES TO EXTERNAL DOOR FRAMES IN EXISTING BUILDING AS SHOWN ON DRAWINGS

-	Quality of wood and fixing: To BS 1186-3.

-	Species: Contractors choice.

-	Class: CSH and 1.

-	Moisture content at time of fixing: 6% to 10%.

-	Preservative treatment: none.

- Profile: To match existing

- -Finished size: To match existing.

-	Finish as delivered: prepared and primed.

-	Fixing: Lost head nail.

P20/200	MEDIUM DENSITY FIBREBOARD FOR INTERNAL WINDOW CILL BOARDS & CAPPING FOR LOW PARTITION AREA G18

-	Manufacturer: Contractor’s choice.

-	Standard: To BS EN 622-5.

-	Type: MDF .

-	Formaldehyde class: not applicable

-	Thickness: 25mm and as drawings .

-	Edges:rounded .

-	Finish: Prepared and primed .

-	Support/ Fixing: To sw grounds with lost nails .

P20/360	PINBOARD (IN ADDITION TO PINBOARD AS PART OF ACOUSTIC WALL ABSORBER)

-	Manufacturer: Celotex

-	Product reference: Sundeala board, fire rated.

-	Thickness: 9 mm.

-	Size: approx 1800mm high x overall 12m long in 4 separate sections in area G13, plus 1200mm high x 4m long in area G20

-	Edges: Ex 50 x 25mm sw framing

-	Decorative facing: None.

P20/510	INSTALLATION GENERALLY

-	Joinery workmanship: As section Z10.

-	Metal workmanship: As section Z11.

-	Methods of fixing and fasteners: As section Z20 where not specified.

-	Straight runs: To be in one piece, or in long lengths with as few joints as possible.

-	Running joints: Location and method of forming to be agreed where not detailed.

-	Joints at angles: Mitre unless shown otherwise.

-	Position and level: To be agreed where not detailed.