N11	DOMESTIC KITCHEN FITTINGS, FURNISHINGS AND EQUIPMENT

To be read with Preliminaries and General conditions.

PRODUCTS

N11/340	WORKTOPS AREAS G06, & G07 (TEA POINT)

-	Classification: TO BS 6222-3, Type 2 .

-	Material: plastic laminate with matching edging, colour to be selected.

-	Dimensions: 600 wide .

-	Support: base units and 19 x 44 battens at walls .

N11/350	SINK, TAPS, TRAP AND WASTE AREAS G06

-	Standard: BS EN 13310 .

-	Manufacturer: Franke or equal approved inset sink.

-	Product reference: ASX611 Propack.

-	Size: 860 x 510 .

-	Material: stainless steel .

-	Single taphole

-	Taps: included in the above reference

-	Wastes: chain and plug .

-	Standards: To BS EN 274-1, -2 and -3.

-	Manufacturer: Contractor’s choice .

-	Size: DN 40.

-	Traps: Tubular .

-	Standards: To BS EN 274-1, -2 and -3.

-	Manufacturer: Contractor’s choice.

-	Size:DN 40.

-	Depth of seal (minimum): 75 mm.

N11/350/A	SINK, TAPS, TRAP AND WASTE AREAS G07

-	Standard: BS EN 13310 .

-	Manufacturer: Franke or equal approved inset sink.

-	Product reference: Rotondo bowl.

-	Size: 395 diameter .

-	Material: stainless steel .

-	No taphole

-	Taps: Monoblock mixer to match item in N11/350

-	Wastes: chain and plug .

-	Standards: To BS EN 274-1, -2 and -3.

-	Manufacturer: Contractor’s choice .

-	Size: DN 40.

-	Traps: Tubular .

-	Standards: To BS EN 274-1, -2 and -3.

-	Manufacturer: Contractor’s choice.

-	Size:DN 40.

-	Depth of seal (minimum): 75 mm.

N11/380	PURPOSE MADE BASE UNITS, SHELVING AND WALL UNITS AREAS G06 AND G07 (TEA POINT)

-	Standard: To BS 6222-2, -3 and -5 and BS EN 14749.

-	Manufacturer: Contractor’s choice .

-	Structural performance: To BS 6222-2, grade H .

-	Surface finishes: To BS 6222-3.

-	Doors and drawer fronts:

-	Material: Birch faced ply .

-	Thickness:18mm .

-	Finish and colour: As section M60 .

-	Edges:unlipped .

-	Other requirements: 180 degree concealed  hinges .

-	Side panels, plinths and shelves:

-	Material: as door and drawer fronTs .

-	Thickness: 18mm .

-	Finish and  colour: As section M60 .

-	Edges: unlipped.

-	Accessories: 2No chrome plated hob guards, Hafele ref 521.25.210.

N11/390	SEALANT POINTING

-	Material: Silicone based to BS 5889, Type B with fungicide.

-	Colour: to  be agreed .

-	Manufacturer: Contractor’s choice ..

-	Application: As section Z22.

EXECUTION

N11/610	MOISTURE CONTENT OF WOOD AND WOOD BASED BOARDS

-	Control and monitoring:

-	Method statement: Submit.

N11/620	INSTALLATION GENERALLY

-	Fixings and adhesives: As section Z20.

N11/630	INSTALLING UNITS AND WORKTOPS

-	General: Well fitting, stable and secure.

N11/640	INSTALLING APPLIANCES

-	Connections: Provide to electric, gas, and hot and cold water services.

N11/650	INSTALLING SINKS, TAPS AND WASTES

-	Water supply: To BS 6700 and BS EN 806-2.

-	Taps:

-	Fixing: Secure, watertight seal with the appliance.

-	Positioning: Hot tap to left of cold tap as viewed by the user of the appliance.

-	Wastes:

-	Bedding: Waterproof jointing compound.

-	Fixing: With resilient washer between appliance and backnut.

N11/660	SEALANT BEDDING AND POINTING

-	Application: As section Z22.

-	Bedding: Bed sinks to top of worktops .

-	Pointing: Joints between units and splashbacks .

COMPLETION

N11/910	GENERAL

-	Doors and drawers: Accurately aligned, not binding. Adjusted to ensure smooth operation.

-	Ironmongery: Checked, adjusted and lubricated to ensure correct functioning.