N17	PORTABLE FIRE FIGHTING SYSTEMS

To be read with Preliminaries/ General conditions.

GENERAL

N17/110	PORTABLE FIRE EXTINGUISHER SYSTEM: 12 NO IN LOCATIONS TO BE AGREED

-	Type: Water .

-	Capacity: 2 litre .

-	Supports: Floor stand – double unit .

N17/110/A	PORTABLE FIRE EXTINGUISHER SYSTEM: 12NO IN LOCATIONS TO BE AGREED

-	Type: Carbon Dioxide .

-	Capacity: 2 kg .

-	Supports:  Floor stand – double unit.

N17/110/B	PORTABLE FIRE EXTINGUISHER SYSTEM: 1NO IN LOCATION TO BE AGREED

-	Type: Foam .

-	Capacity: 2 kg .

-	Supports:  Floor stand – double unit.

N17/150	FIRE BLANKET SYSTEM 1NO IN LOCATION TO BE AGREED

-	Type: To BS EN 1869 .

-	Size: 1.2 x 1.2m .

-	Supports: Mounting bracket.

SYSTEM PERFORMANCE

N17/220	COLOUR CODING

-	Portable fire extinguishers: Colour code in accordance with BS 7863.

PRODUCTS

N17/310	CARBON DIOXIDE EXTINGUISHERS

-	Standard: To BS EN 3-6.

-	Manufacturer: Chubb or equal approved.

N17/310	FOAM EXTINGUISHERS

-	Standard: To BS EN 3-6.

-	Manufacturer: Chubb or equal approved.

N17/340	WATER EXTINGUISHERS

-	Standard: To BS EN 3-6.

-	Manufacturer: Chubb or equal approved.

N17/410	FIRE BLANKETS - LIGHT DUTY

-	Standard: To BS EN 1869.

-	Manufacturer: Chubb or equal approved.

EXECUTION

N17/610	INSTALLING PORTABLE FIRE EXTINGUISHERS

-	Mounting height above finished floor level: to be agreed .

N17/650	INSTALLING FIRE BLANKETS

-	Mounting height above finished floor level: to be agreed .

COMPLETION

N17/910	CLEANING

-	Protective wrappings: Remove.

-	Cleaning: Clean off and wipe down container finishes.

N17/920	TESTING

-	Test standard: To BS 5603-0.

N17/930	TRAINING

-	Training: Submit instruction manuals or supply other appropriate resources to train the users of the building in the safe and appropriate use of the fire extinguishers and fire blankets.

-	Fire brigade: Submit contact details.

N17/940	MAINTENANCE

-	Servicing: Arrange the first annual service of the portable fire fighting systems.

-	Maintenance standard: To BS 5603-0.