# Q40 Fencing

To be read with Preliminaries/ General conditions.

FENCING SYSTEMS

410A	PURPOSE MADE FENCINGTO  NURSERY PLAYGROUND

•	 Standard: To BS 1722- 9 except as varied on drawings and below:

•	 Height: Refer to drawings.

•	 Materials: Mild steel.

-	Treatment: Hot dip galvanized to BS EN ISO 1461.

•	 Centres of posts (maximum): Refer to structural engineers drawings.

•	 Method of setting posts: Refer to structural engineers drawings.

•	 Accessories: Single leaf gate as shown on drawings, including ironmongery.

410B	PURPOSE MADE FENCINGTO FRONT ELEVATION AT BACK EDGE OF PAVEMENT AND TO BIKE AND BIN STORES

•	 Standard: To BS 1722- 9 except as varied on drawings and below:

•	 Height: Refer to drawings.

•	 Materials: Mild steel.

-	Treatment: Hot dip galvanized to BS EN ISO 1461, etched and painted.

•	 Centres of posts (maximum): Refer to structural engineers drawings.

•	 Method of setting posts: Refer to structural engineers drawings.

•	 Accessories: 5no single leaf gates and 2no. double leaf gates as shown on drawings,

including all ironmongery; nb: bike store gates to have electronic access control - see M&E spec.

Other requirements: Fence panels and gates to incorporate solid panels where shown;

2no. main gates to incorporate panels laser cut with designs created from childrens

drawings; LAYCOCK PRIMARY SCHOOL to be formed from laser cut steel sheet

fabricated into 50mm thick hollow box forms and set into the railings (refer to details)

410C	PURPOSE MADE FENCINGTO RAMPS ON EAST ELEVATION AND AT FRONT

ENTRANCE, AND TO STEPS ON EAST ELEVATION

•	 Standard: To BS 1722- 9 except as varied on drawings and below:

•	 Height: Refer to drawings.

•	 Materials: Mild steel.

-	Treatment: Hot dip galvanized to BS EN ISO 1461, etched and painted.

•	 Centres of posts (maximum): Refer to structural engineers drawings.

•	 Method of setting posts: Refer to structural engineers drawings.

•	 Accessories: none (see L30/570 for handrails).

EXECUTION

710	INSTALLATION GENERALLY

•	 Set out and erect:

-	Alignment: Straight lines or smoothly flowing curves.

-	Tops of posts: Following profile of the ground.

-	Setting posts: Rigid, plumb and to specified depth, or greater where necessary

to ensure adequate support.

-	Fixings: All components securely fixed.

720	SETTING POSTS IN CONCRETE

•	 Standard: To BS 8500-2.

•	 Mix: Designated concrete not less than GEN1 or Standard prescribed concrete not

less than ST2.

•	 Admixtures: Do not use.

•	 Holes: Excavate neatly and with vertical sides.

•	 Filling: Position post/ strut and fill hole with concrete to not less than the specified

depth, well rammed as filling proceeds and consolidated.

•	 Backfilling of holes not completely filled with concrete: Excavated material, well

rammed and consolidated.

730	EXPOSED CONCRETE FOUNDATIONS

•	 Filling: Compact until air bubbles cease to appear on the upper surface.

•	 Finishing: Weathered to shed water and trowelled smooth.

780	DAMAGE TO GALVANIZED SURFACES

•	 Treatment of minor damage (including on fasteners and fittings): Low melting point

zinc alloy repair rods or powders made for this purpose, or at least two coats of

zinc-rich paint to BS 4652.

•	 Thickness: Apply sufficient material to provide a zinc coating at least equal in

thickness to the original layer.