N13	SANITARY APPLIANCES AND FITTINGS

To be read with Preliminaries/ General conditions.

TYPES OF APPLIANCE/ FITTING

N13/300	WC’S AND CISTERNS FOR ALL BOYS, GIRLS AND NURSERY TOILETS

-	Standard: To DEFRA WC suite performance specification or approved by relevant water company.

-	Type: Back to wall with concealed dual flush low level cistern

- Pan:

-	Standards: Vitreous china to BS 3402.

-	Manufacture : Armitage Shanks.

Product reference: htm 64 junior profile WC .

-	Material: Vitreous china, white.

-	Seat and cover:

-	Manufacturer: Armitage Shanks ref S407401

-	Flushing arrangement: Plastic diaphragm type float operated valve and siphon to BS 1212-3 and float to B 2456.

- Operating control: Single push button.

N13/301	WC’S AND CISTERNS FOR STAFF TOILETS

-	Standard: To DEFRA WC suite performance specification or approved by relevant water company.

- Type: Back to wall with concealed dual flush low level cistern

- Pan:

-	Standards: Vitreous china to BS 3402.

-	Manufacture : Contractors choice.

-	Material: Vitreous china, white.

-	Seat and cover:

-	Material: Plastic, White

-	Flushing arrangement:

-	Operating control: Single push button.

N13/311	DOCUMENT M PACKAGES FOR ACCESSIBLE WC’S AREAS G15, F07 AND S07

-	Manufacturer: Armitage Shanks Special Care Doc M Standard pack with all components

-	Product reference: S687501.

-	Type approval certificate: Submit.

N13/315	URINALS AND AUTOFLUSHING CISTERNS AREAS G08 AND G18

-	Urinals:

-	Standard: To BS 4880-1

-	Manufacturer: Pland or equal approved .

Product reference: Lyon, floor recessed slab urinal.

-	Material: Stainless steel .

-	Wastes: to suit .

-	Standards: To BS EN 274-1, -2 and -3.

-	Manufacturer: Contractor’s choice .

-	Size: DN 40 ..

-	Tail: Unslotted.

-	Traps: .

-	Standards: To BS EN 274-1, -2 and -3.

-	Manufacturer: Contractor’s choice .

-	Size: DN40.

-	Depth of seal (minimum): 75 mm.

-	Cistern, complete with automatic siphon, lid, supports and fixings: .

-	Standard: To BS 1876.

-	Manufacturer: Pland.

-	Flush volume: 4.5l .

-	Operating control: Proximity sensor .

-	Flush pipe: Exposed .

Material: Chrome .

N13/331	SINKS FOR STORAGE/WET AREAS IN NURSERY AREA G20

-	Standard: BS EN 13310 .

-	Manufacturer: Sissons or equal approved .

-	Product reference: B2 0085R & B2 0086D.

-	Size: 1015 x 505 & 1800 x 500.

-	Material: stainless steel .

-	Taps/ Chainstay/ Overflow holes: 2 tapholes, chain and overflow .

-	Water supply fittings: High neck pillar taps.

-	Manufacturer: Sissons or equal approved .

Product reference: F1083.

-	Wastes: chain and plug .

-	Standards: To BS EN 274-1, -2 and -3.

-	Manufacturer: Contractor’s choice .

-	Size: DN 40.

-	Traps: Tubular .

-	Standards: To BS EN 274-1, -2 and -3.

-	Manufacturer: Contractor’s choice.

-	Size:DN 40.

-	Depth of seal (minimum): 75 mm.

N13/331/A	SINKS FOR STORAGE/WET AREAS IN CLASSROOMS AND UNIT ROOMS AREAS G25, S01, S12, S18, S21 AND S28

-	Standard: BS EN 13310 .

-	Manufacturer: Sissons or equal approved.

-	Product reference: B2 0085R & C20120N.

-	Size: 1015 x 505 & 411 x 340.

-	Material: stainless steel .

-	TapS/ Chainstay/ Overflow holes: 2 tapholes, chain and overflow .

-	Water supply fittings: High neck pillar taps .

-	Manufacturer: Sissons or equal approved.

Product reference: F1083.

-	Wastes: chain and plug .

-	Standards: To BS EN 274-1, -2 and -3.

-	Manufacturer: Contractor’s choice .

-	Size: DN 40.

-	Traps: Tubular .

-	Standards: To BS EN 274-1, -2 and -3.

-	Manufacturer: Contractor’s choice.

-	Size:DN 40.

-	Depth of seal (minimum): 75 mm.

N13/335	WASH BASINS FOR CHILDREN AREA G17

-	Manufacturer: TC Express or equal approved  .

-	Product reference: Semi-recessed basin .

-	Size: 450 x 350mm .

-	Material: vitreous china, white .

-	Tap/ Chainstay/ Overflow holes: 2 tapholes, chainstay and overflow hole .

-	Water supply fittings: pilar taps .

-	Manufacturer: Armitage Shanks or equal approved.

Product reference: Avon or equal approved .

-	Operation: Manual, non-concussive .

-	Wastes: chain and plug .

-	Standards: To BS EN 274-1, -2 and -3.

-	Manufacturer: Contractor’s choice.

-	Size: DN 40. .

-	Traps: Tubular .

-	Standards: To BS EN 274-1, -2 and -3.

-	Manufacturer Contractor’s choice.

-	Size DN 40..

-	Depth of seal (minimum): 75 mm.

N13/335/A	WASH BASINS FOR CHILDREN IN ALL OTHER AREAS

-	Manufacturer: Ideal Standard  .

-	Product reference: Studio 35 .

-	Size: 350 x 250mm .

-	Material: vitreous china, white .

-	Tap/ Chainstay/ Overflow holes: 2 tapholes, chainstay and overflow hole .

-	Water supply fittings: pilar taps .

-	Manufacturer: Armitage Shanks or equal approved.

Product reference: Avon or equal approved .

-	Operation: Manual, non-concussive .

-	Wastes: chain and plug .

-	Standards: To BS EN 274-1, -2 and -3.

-	Manufacturer: Contractor’s choice.

-	Size: DN 40. .

-	Traps: Tubular .

-	Standards: To BS EN 274-1, -2 and -3.

-	Manufacturer Contractor’s choice.

-	Size DN 40..

-	Depth of seal (minimum): 75 mm.

N13/335/B	WASH BASINS FOR STAFF

-	Manufacturer: Ideal Standard  .

-	Product reference: Studio 45 .

-	Size: 450 x 300mm .

-	Material: vitreous china, white .

-	Tap/ Chainstay/ Overflow holes: 2 tapholes, chainstay and overflow hole .

-	Water supply fittings: pilar taps .

-	Manufacturer: Armitage Shanks or equal approved.

Product reference: Avon or equal approved manual non-concussive

-	Wastes: chain and plug .

-	Standards: To BS EN 274-1, -2 and -3.

-	Manufacturer: Contractor’s choice.

-	Size: DN 40. .

-	Traps: Tubular .

-	Standards: To BS EN 274-1, -2 and -3.

-	Manufacturer Contractor’s choice.

-	Size DN 40.

-	Depth of seal (minimum): 75 mm.

N13/375	SHOWER UNIT AREA G15

-	Shower fittings: .

-	Manufacturer: Trevi Showers or equal approved .

Product reference: L6736AA Trevi CTV built-in concentric thermostatic valve with 3 function gravity handset shower kit, chrome plated, or equal approved .

-	Wastes: Top access .

-	Standards: To BS EN 274-1, -2 and -3.

-	Manufacturer: Contractor’s choice.

-	Size: DN 40. .

-	Traps: Tubular .

-	Standards: To BS EN 274-1, -2 and -3.

-	Manufacturer: Contractor’s choice.

-	Size: DN 40. .

-	Depth of seal (minimum): 50 mm.

-	Accessories: Pressalit adjustable height nursing shower bench, ref 09 117 4721, 1300mm

N13/580	SEALANT FOR POINTING

-	Standard: To BE EN ISO 11600 .

-	Class: F20 HM .

-	Type: .

-	Manufacturer: Contractor’s choice..

-	Colour: White .

EXECUTION

N13/610	INSTALLATION GENERALLY

-	Assembly and fixing: Surfaces designed to falls to drain as intended.

-	Fasteners: Nonferrous or stainless steel.

-	Supply and discharge pipework: Fix before appliances.

-	Fixing: Fix appliances securely to structure. Do not support on pipework.

-	Jointing and bedding compounds: Recommended by manufacturers of appliances, accessories and pipes being jointed or bedded.

-	Appliances: Do not use. Do not stand on appliances.

-	On completion: Components and accessories working correctly with no leaks.

-	Labels and stickers: Removed.

N13/620	NOGGINGS AND BEARERS

-	Noggings, bearers, etc. to support sanitary appliances and fittings: Position accurately. Fix securely.

N13/630	TILED BACKGROUNDS OTHER THAN SPLASHBACKS

-	Timing: Complete before fixing appliances.

-	Fixing appliances: Do not overstress tiles.

N13/660	INSTALLING SLAB URINALS

-	Waterproofing of walls and floor (specified elsewhere): Completed before fixing urinal components.

-	Gap between components: 3 mm.

-	Space behind channels and slabs: Grout with 1:5 cement:sand grout.

-	Pointing: Rake out joints to 10 mm depth. Point flush with waterproof jointing compound recommended by urinal manufacturer.

N13/670	INSTALLING CISTERNS

-	Cistern operating components: Obtain from cistern manufacturer.

-	Float operated valve: Matched to pressure of water supply.

-	Overflow pipe: Fixed to falls and located to give visible warning of discharge.

-	Location: Agreed, where not shown on drawings.

N13/710	INSTALLING TAPS

-	Fixing: Secure against twisting.

-	Seal with appliance: Watertight.

-	Positioning: Hot tap to left of cold tap as viewed by user of appliance.

N13/720	INSTALLING WASTES AND OVERFLOWS

-	Bedding: Waterproof jointing compound.

-	Fixing: With resilient washer between appliance and backnut.

N13/755	SEALANT BEDDING AND POINTING

-	Bedding: Bed sinks to worktops where appropriate .

-	Pointing: Joints between appliances and walls/splashbacks .