L10	WINDOWS/ ROOFLIGHTS/ SCREENS/ LOUVRES

To be read with Preliminaries/ General conditions.

GENERAL INFORMATION/ REQUIREMENTS

L10/110	EVIDENCE OF PERFORMANCE

-	Certification: Provide independently certified evidence that all incorporated components comply with specified performance requirements.

L10/120	SITE DIMENSIONS

-	Procedure: Before starting work on designated items take site dimensions, record on shop drawings and use to ensure accurate fabrication.

-	Designated items: All openings to receive new windows.

COMPONENTS

L10/250	PURPOSE MADE WOOD WINDOWS WF39C AND WS34C, AND PART OF COMBINED DOOR/WINDOW ASSEMBLIES DG05A, DG11B/C, DG23 & DG23

-	Standard: To BS 644.

-	Manufacturer: A firm currently registered under a third party quality assurance scheme.

-	Operation and strength characteristics: To BS 6375-2.

-	Timber: Generally to BS EN 942.

-	Species: Contractors choice

-	Appearance class: J10 for glazing beads, drip mouldings and the like. J40 or better for all other members.

-	Moisture content on delivery: 12-19%.

-	Preservative treatment: Organic solvent as Z12

-	Finish as delivered: Primed

-	Glazing details: To achieve U value of max. 1.8 W/sqmK. Profiled fixing beads, see details

-	Ironmongery/ Accessories: Teleflex Morse remote opening gear for opening casements.

-	Fixing: Galvanised cramps.

L10/400	VELFAC 200 SERIES COMPOSITE WINDOWS (ALL OTHER NEW WINDOWS, SEE DRAWINGS)

Manufacturer:

- 	VELFAC Ltd, The Old Livery, Hildersham, Cambridge, CB1 6DR Telephone 01223 897100 Fax 01223 897101

Email ijo@VELFAC.co.uk

Contact – Iain Jones 07825 059 149

Product reference:

- VELFAC 200 Series Composite Windows

- Secured By Design (SBD) approved V210 Fixed and V212 Tophung Projecting

Windows, curtain walling and other glazed areas are to be a unitised composite aluminium/wood system with a uniform appearance,

i.e. fixed lights have same sight-lines as opening lights.

V200 Fire Windows - Manufactured from profiles providing uniformity of sightlines across all functions and to be visually identical to adjacent elements.

Windows to be tested to BS EN 1363-1, BS EN 1363-2, BS EN 1364-1 and provide class E60 (integrity) fire resistance in the following functions: V250 Fixed window & V272 Top hung projecting window.

V229 Reversible opening window:
Manufactured from profiles providing uniformity of sightlines across all functions. Sashes mounted on aluminium guide rails with balance arms incorporating built in adjustable friction brake and opening safety restrictor. Window to be fully reversible to facilitate internal cleaning and include automatic engagement of safety catch when placed into the cleaning position.

- The system must be a fully ventilated and drained system with concealed drain-channels, concealed ironmongery and prefabricated.

Softwood windows with an external aluminium cladding will not be permitted.

System should carry a 12 year manufacturer warranty

System should have an expected life span of 40 years with regular maintenance

Materials:

Exterior frame/ sash cladding:  54mm wide external sash

With an external aluminium sash alloy AIMgS0,5.

Finish:

The external aluminium sash must be polyester powder-coated in compliance with BS 6496, BS 6497, GSB and DIN 50939, in RAL colour to gloss level 77% and a primary coating thickness of between 60-120 µm

Colour to be chosen from the RAL Classic standard colour range

Interior frame section:

The system must have an internal structural frame of laminated Scandinavian Pinewood (Pinus Sylvestris),

Dimensions :50mmx90mm and  50mmx115mm

VACUMAT- treated pine with water based lacquer containing anti-mould fungicide.

Cill groove type 26. No. G378-26.

Bevel type 378090-1 to corner units.

Corner post type A2001.

Standard vertical couple to architects details.  VELFAC No. CD200-200-V01.

Vertical coupling details to incorporate drip flashing as per architects details. VELFAC No. CD200-200-V02.

Glazing details:

The glazing units are to be 24mm thick double glazed units comprising nominally of

6mm toughened super low e inner / 14mm argon filled cavity / 6.8mm laminated outer.

Ironmongery/ Accessories:

Opening handles are to be single handles incorporating a multipoint locking mechanism fully concealed within the sash with only one handle permitted per window

Code 25 key lockable espagnolette handle

Satin anodised aluminium operating multi-point locking gear

Code 13 100mm Locking allen key restrictors

Trickle vents where applicable

Fixing:

The windows are to be fully and permanently sealed to the wall construction around using Arbosil 1090 Silicon Sealant or Compriband 600 Joint Sealing Tape in order to maintain an impervious barrier throughout. The sealing tape is to be fixed to the perimeter of all windows, sealed continuously to both the windows, wall substrate and connecting mullions (where applicable), in order not to interfere with the window systems drainage path – all as per manufacturers instructions / recommendations.

Exposure category to BS 6375-1/ Design wind load:

The window system must meet the air permeability requirements of test pressure class 600PA given in BS 6375 part 1, and no deformation or damage must be recorded at an applied wind pressure of 2400PA.

The window system must meet the weather tightness requirements of test pressure class 300 given in BS 6375 part 1, and the window system must be tested to meet the requirements of BS 6375 part 2.

Operation and strength characteristics: To BS 6375-2.

L10/510	GLAZED WOOD SCREENS (SEE DRAWINGS)

-	Timber: Generally to BS EN 942.

-	Species: Contractors choice

-	Moisture content on delivery: 9-13%. .

-	Panels: 22mm MDF

-	Assembly adhesive: PVAC to BS EN 204, Class D4

-	Joinery workmanship: As section Z10.

-	Finish as delivered: Primed

-	Glazing details: single glazed panels see drawings.

-	Fixing: Galvanised cramps and screws .

L10/460	ROOFLIGHTS R01, R02, R03 & R-4

-	Manufacturer: National Domelight Company, tel  01276 451555.

-	Product reference: Thernalight Fixed (standard) for R01, R03 and R03, Thermalite Hinged (manual) for R04.

-	Type: Thernalight Fixed (standard)  .

-	Frame: Extruded thermally broken aluminium profile, grade 6063 T6 .

-	Finish: Polyester powder coated .

-	Colour: to be confirmed .

-	Kerb: 75mm wide up-stand to all sides as manufacturers recommended. Minimum 3 degrees fall .

-Glazing details: Fixed: 28mm DGU comprising 6mm clear toughened outer pane,

16mm airspace and 6mm clear toughened Low E coated inner pane. Units to have

silicone edged seal .  Hinged: 32mm DGU comprising 6mm clear toughened outer

pane, 20mm airspace and 6mm clear toughened Low E coated inner pane. Units to

have silicone edged seal.

-	Other requirements: External glazing surfaces to be flush with no flashings or water ponding obstructions/dirt traps on the glass surface. .

-	Fixing: As manufacturer’s recommendations .

650	METAL LOUVRES – BASEMENT VENT IN NURSERY PLAYGROUND

•  Manufacturer: Contractor's choice.

-	Product reference: TBA .

•	 Material: Steel .

-	Finish as delivered: Galvanized .

•	 Fire resistance rating: Not applicable .

•	 Number of louvre banks: To suit .

•	 Louvre blade pitch and angle: To match existing .

•	 Blanking panels: Not required .

•	 Accessories/ Other requirements: Stainless steel bird mesh .

•	 Fixing: Plug and screw to brickwork .

- 650/A	METAL LOUVRES – SUBSTATION, AS REQUIRED BY EDF

•  Manufacturer: Sunray Engineering.

-	Product reference: Type Q .

•	 Material: Steel .

-	Finish as delivered: Polyester powder coated .

•	 Fire resistance rating: Not applicable .

•	 Number of louvre banks: To suit .

•	 Louvre blade pitch and angle: To suit .

•	 Blanking panels: Not required .

•	 Accessories/ Other requirements: Fine mesh backing screen .

•	 Fixing: As manufacturer’s recommendations .

INSTALLATION

L10/710	PROTECTION OF COMPONENTS

-	General: Do not deliver to site components that cannot be installed immediately or placed in clean, dry floored and covered storage.

-	Stored components: Stack vertical or near vertical on level bearers, separated with spacers to prevent damage by and to projecting ironmongery, beads, etc.

L10/730	PRIMING/ SEALING

-	Wood surfaces inaccessible after installation: Prime or seal as specified before fixing components.

L10/750	BUILDING IN

-	General: Not permitted unless indicated on drawings.

-	Brace and protect components to prevent distortion and damage during construction of adjacent structure.

L10/765	WINDOW INSTALLATION GENERALLY

-	Installation: Into prepared openings.

-	Gap between frame edge and surrounding construction:

-	Minimum: 6mm

-	Maximum: 12mm .

-	Distortion: Install windows without twist or diagonal racking.

L10/780	FIXING OF WOOD FRAMES

-	Standard: As section Z20.

-	Fasteners: Galvanised cramps and screws

-	Spacing: When not predrilled or specified otherwise, position fasteners not more than 150 mm from ends of each jamb, adjacent to each hanging point of opening lights, and at maximum 450 mm centres.

L10/784	FIXING OF COMPOSITE FRAMES

-	Standard: As section Z20.

-	Fasteners: Galvanised cramps and screws

-	Spacing: When not predrilled or specified otherwise, position fasteners not more than 150 mm from ends of each jamb, adjacent to each hanging point of opening lights, and at maximum 600 mm centres.

L10/790	FIRE RESISTING FRAMES

-	Gap between back of frame and reveal: Completely fill with intumescent putty.

L10/810	SEALANT JOINTS

-	Sealant:

-	Manufacturer: Contractors choice

-	Colour: To be advised

-	Application: As section Z22 to prepared joints. Finish triangular fillets to a flat or slightly convex profile.

L10/820	IRONMONGERY

-	Fixing: Assemble and fix carefully and accurately using fasteners with matching finish supplied by ironmongery manufacturer. Do not damage ironmongery and adjacent surfaces.

-	Checking/ Adjusting/ Lubricating: Carry out at completion and ensure correct functioning.