K40	DEMOUNTABLE SUSPENDED CEILINGS

To be read with Preliminaries/ General conditions.

TYPES OF CEILING SYSTEM

K40/110	UNIT SUSPENDED CEILING SYSTEMS WHERE SHOWN AS ACOUSTIC CEILING PANLES ON DRAWINGS.

-	Structural soffits: Concrete

- Ceiling system manufacturer: Armstrong World Industries,

Freephone 0800 371849, fax 01895 274287

-	Product reference: Orcal D-H 700 Microperforated with double black scrim faced mineral wool acoustic infill

-	Ceiling type:

-	Grid form: U-Profile

-	Grid exposure: Concealed Grid

-	Grid spacings: TBC

-	Access: Infill Units Fully Demountable

-	Ceiling module: TBC

-	Ceiling soffit above finished floor level: approx 2750 mm (varies where waste pipes are located at high level in rooms).

-	Components:

-	Suspension system: As clause 217

-	Perimeter trim: Not required

-	Timber edge battens: Not required

-	Infill units: As clause 221

-	Access units: Not required

-	Insulation: As clause 226 (if required)

-	Air plenum barriers: Not required

-	Accessories: As clause 228 (if required)

-	Integrated services fittings: not applicable.

-	Other requirements: To be installed as per drawing reference Sys 5

GENERAL/PERFORMANCE

K40/203	DEFINITIONS

-	Ceiling: Items collectively constituting the ceiling surface i.e. infill units/boards/stretched fabric, access units and grid.

-	Ceiling system: Ceiling plus suspension system and integrated services fittings.

COMPONENTS

K40/217	SUSPENSION SYSTEM D-H 700 Suspension System

-	Extent of system: Include U-Profile, cross connectors, plug in clips and hanger brackets

-	Materials:

-	Top fixings: Non-specified

- Hangers: 6mm Threaded Rod

- Grid type: Concealed Grid System

-	Finish: Galvanised

-	Colour: Self coloured

K40/218	TIMBER EDGE BATTENS – n/a : floating solution

K40/219	PERIMETER TRIMS – n/a : floating solution

K40/221	INFILL UNITS Armstrong D-H 700 Panels

-	Type: Steel Tile

-	Form: Downwardly de-mountable tiles

-	Sizes: Various, see drawing ACS SYS-5 DH700

-	Finish: Polyester Powder Coat

-	Colour: RAL 9010

K40/226	INSULATION (LOOSE LAID)

-	Type: Rockwool

-	Thickness: 25 mm.

-	Density: 45 kg/m³.

-	Facings/wrappings: Black tissue facing to both front and reverse faces.

K40/227	AIR PLENUM BARRIERS  – n/a : floating solution

INSTALLATION

K40/302	CONTROL SAMPLES

-	General: Complete areas of the finished work in the following locations: to be agreed with contractor.

K40/305 	SETTING OUT

-	General: Set out accurately, continuous, even, and jointed at regular intervals. Provide level soffits free from undulations, lipping and distortions in grid members.

-	Infill and access units, integrated services: Fit correctly and aligned.

-	Edge/perimeter infill units size (minimum): Half standard width or length.

-	Grid: Position to suit infill unit sizes. Allow for permitted deviations from nominal sizes.

-	Infill joints and exposed suspension members: Straight, aligned and parallel to walls. Where building elements and features to which the ceiling systems relate are not square, straight or level, give notice.

K40/310	BRACING

-	General: Secure, with additional bracing and stiffening as necessary to give a stable ceiling system resistant to design loads and pressures.

K40/315	PROTECTION

-	Loading: Do not apply loads for which the suspension system is not designed.

-	Ceiling materials: Remove and replace correctly using special tools and clean gloves, etc. as appropriate.

K40/350	TOP FIXING

-	Building structure: Verify suitability.

-	Concrete: Drill and insert suitable expanding anchors.

-	Aerated concrete: Fix through from the top of the concrete unit and provide a system of primary channels.

-	Structural steel: Drill, or use suitable proprietary clips/adaptors.

-	Metal roof decking: Fix to sides of corrugations.

-	Timber: Fix to side of joists at least 50 mm from bottom edge. If ceiling system is intended for fire protection, fix into top third of joists.

-	Hollow structural members: Submit fixing proposals.

-	Cartridge or powder activated methods: Do not use.

K40/360 	INSTALLING HANGERS

-	Rigid hangers:

-	Installation: Install vertical or near vertical. Do not allow hangers to press against fittings, services, insulation covering ducts/pipes.

-	Obstructions: Where obstructions prevent vertical installation, either:

-	brace diagonal hangers against lateral movement; or

-	hang ceiling system on an appropriate rigid sub-grid bridging across obstructions and supported to prevent lateral movement.

-	Extra hangers: Provide as required to carry additional loads.

-	Fixing:

-	Angle/strap hangers: Do not rivet for top fixing.

-	Spacings: As manufacturer’s recommendations.

K40/370 	INSTALLING TIMBER EDGE BATTENS

- 	n/a : floating solution

K40/380	INSTALLING PERIMETER TRIMS

- 	n/a : floating solution.

K40/382	EXPOSED GRIDS

- 	n/a : floating solution

K40/384	CONCEALED GRIDS

- 	n/a : floating solution

K40/386	INSTALLING INFILL UNITS

-	Concealed grids: Install infill units uniform, straight and aligned. Avoid dimension creep.

-	Infill units around recessed luminaires and similar openings: Prevent movement and displacement.

K40/387	INSTALLING METAL INFILL UNITS

-	Removal: Do not damage infill units. Use tools designed for the purpose.

K40/389	UPSTANDS AND BULKHEADS

-	Vertical ceiling systems: Support and brace to provide alignment and stability.

-	High upstands: Provide support at base of upstand.

K40/390	OPENINGS IN CEILING MATERIALS

-	Forming: Neat and accurate. Suit sizes and edge details of fittings. Do not distort ceiling system.

K40/397	TRUNKING

-	Recessed trunking: Provide flanges for support of grid and infill units, unless mounted above grid flanges. Retain in place with lateral restraint.

K40/400 	INSTALLING INSULATION

-	Fitting: Fit accurately and firmly with closely butted joints and no gaps.

-	Insulation within individual infill units: Fit closely. Secure to prevent displacement when infill units are installed or subsequently lifted.

-	Width: Lay insulation in the widest practical widths to suit grid member spacings.

-	Services: Do not cover electrical cables not sized accordingly. Cut insulation carefully around electrical fittings, etc. Do not lay insulation over luminaires.

-	Sloping and vertical areas of ceiling system: Fasten insulation to prevent displacement.

COMPLETION

K40/520	USER INSTRUCTIONS

-	Contents: Include the following:

-	Correct methods for removing and replacing infill units and other components.

-	Cleaning methods and materials.

-	Recommendations for redecoration.

-	Ceiling systems intended for fire protection: Limitations placed on subsequent alterations and maintenance procedures, to ensure that their fire performance is not impaired.

-	Maximum number, position and value of point loads that can be applied to ceiling system after installation.