K10	PLASTERBOARD DRY LININGS/ PARTITIONS/ CEILINGS

To be read with Preliminaries/ General conditions.

TYPES OF DRY LINING

K10/125	METAL STUD PARTITION SYSTEM – ALL NEW PARTITIONS	-	Manufacturer: British Gypsum .

-    Product reference: 146mm Gypframe AcouStuds

-	Type: 146 AS 50 Acoustic studs

-	Centres: 600c/c’s max

-	Head condition: Existing concrete decks or new steel lintels .

-	Deflection allowance: 10mm maximum

-	Insulation: British Gypsum Isowool  APR1200 acoustic blanket .

-	Thickness: 50mm

-	Linings: 2 layers each face of stud with 15mm British Gysum Soundbloc board

-	Finishing: British Gysum Thistle board finish 2-3mm .

-	Primer/ Sealer: As recommended by manufacturer

-	Accessories: Galvanised skim beads and the like .

-	Other requirements: Seal at all wall, floor and door jamb conditions with British Gypsum Gyproc sealant

K10/165	WALL LINING SYSTEM (METAL FRAMING) TO EXISTING WALL AREAS G01, G02, G06, G07, G09, G13, G17 AND G20, ABOVE EXISTING PLINTH

-	Manufacturer: British Gypsum .

-	Product reference: Gypframe 70S 60 ‘C’ metal Stud

-	Wall: Brickwork

-	Stud centres: 400 c/c’s .

-	Linings: Single layer 15mm  British Gypsum Gyproc Duraline MR board

-	Finishing: Dry line to receive finish as elsewhere .

-	Primer/ Sealer: As recommended by manufacturer

-	Accessories: Ceiling and floor Gypframe fixing channels.

-	Other requirements: Seal all junctions with with British Gypsum Gyproc sealant.

K10/165/A	WALL LINING SYSTEM (METAL FRAMING) TO WALLS SHOWN AS HAVING ACOUSTIC ABSORBERS, FROM 1100MM ABOVE FFL UP TO CEILING, EXCEPT FROM 1700MM/2100MM ABOVE FFL UP TO CEILING WHERE SHOWN ABOVE CUPBOARDS/DOORS

-	Manufacturer: British Gypsum .

-	Product reference: Gyplyner universal walling system

-	Wall: Brickwork/plaster and Soundbloc board on metal studs

-	Stud centres: to suit board width.

-	Linings: 12.5mm wallboard plus 9mm Sundeala FRB dyed Eco board up to 1700mm above FFL, and  British Gypsum 12.5mm Gyptone Quattro 41 boards above.

-	Finishing: Tape and fill Gypsum joints .

-	Primer/ Sealer: As recommended by manufacturer

-	Accessories: Ceiling and floor Gypframe fixing channels.

-	Other requirements: Seal all junctions with with British Gypsum Gyproc sealant, fill voids with British Gypsum Isowool APR1200 acoustic blanket

K10/205	LINING ON TIMBER TO SVP ENCLOSURES AND THE LIKE

-	Background: Timber battens and noggers

-	Linings: As adjacent walls.

-	Fixing: Galvanised drywall screw

-	Finishing: Dry lined .

-	Primer/ Sealer: As recommended by Manufacturer

-	Other requirements: Trim for rodding access to SVP’s .

K10/245	CEILING LINING ON TIMBER TO AREAS G01, G02, G03, G04, G09, G15, & G17

-	Background: Timber joists

-	Metal resilient (acoustic) bars: not required

-	Linings: 12.5mm British Gypsum Duraline MR .

-	Fixings: As manufacturer’s recommendations.

-	Finishing: Seamless jointing

-	Primer/ Sealer: As recommended by manufacturer

-	Accessories: Angle and bead reinforcement where necessary .

-	Other requirements: Seal all junctions with  British Gypsum Gyproc sealant

K10/245/A	CEILING LINING ON TIMBER TO AREAS G06, G07 & G20

-	Background: Timber joists

-	Metal resilient (acoustic) bars: not required

-	Linings: 12.5mm British Gypsum Gyptone Quattro 41 boards and fleece .

-	Fixings: As manufacturer’s recommendations .

-	Finishing: Seamless jointing

-	Primer/ Sealer: As recommended by manufacturer

-	Accessories: Angle and bead reinforcement where necessary .

-	Other requirements: Seal all junctions with  British Gypsum Gyproc sealant.  45mm battens fixed to underside of joists, and glasswool TS48 in cavity.

K10/275	ENCASEMENT ON TIMBER FRAMING TO EXPOSED STEEL BEAMS

-	Timber framework: 44 x 44mm wrot timber with noggins at 600mm max centres..

-	Linings: 2 layers of 12.5mm Fireline board .

-	Fixing: As manufacturer’s recommendations .

-	Finishing: As manufacturer’s recommendations to achieve 1 hour fire resistance .

-	Primer/ Sealer: As manufacturer’s recommendations .

INSTALLATION

K10/335	ADDITIONAL SUPPORTS

-	Framing: Accurately position and securely fix to give full support to:

-	Partition heads running parallel with, but offset from main structural supports.

-	Fixtures, fittings and service outlets. Mark framing positions clearly and accurately on linings.

-	Board edges and lining perimeters, as recommended by board manufacturer to suit type and performance of lining.

K10/375	NEW WET LAID BASES

-	Dpcs: Install under full width of partitions/ freestanding wall linings.

-	Material: Bituminous sheet or plastics.

K10/435	DRY LININGS GENERALLY

-	General: Use fixing, jointing, sealing and finishing materials, components and installation methods recommended by board manufacturer.

-	Cutting plasterboards: Neatly and accurately without damaging core or tearing paper facing.

-	Cut edges: Minimize, and position at internal angles wherever possible. Mask with bound edges of adjacent boards at external corners.

-	Fixing boards: Securely and firmly to suitably prepared and accurately levelled backgrounds.

-	Finishing: Neatly to give flush, smooth, flat surfaces free from bowing and abrupt changes of level.

K10/445	CEILINGS

-	Sequence: Fix boards to ceilings before dry lined walls and partitions.

-	Orientation of boards: Fix with bound edges at right angles to supports and with ends staggered in adjacent rows.

-	Two layer boarding: Stagger joints between layers.

K10/455	METAL FRAMING FOR PARTITIONS/ WALL LININGS

-	Setting out: Accurately aligned and plumb.

-	Frame/ Stud positions: Equal centres to suit specified linings, maintaining sequence across openings.

-	Additional studs: To support vertical edges of boards.

-	Fixing centres at perimeters (maximum): 600 mm.

-	Openings: Form accurately.

-	Doorsets: Use sleeved or boxed metal studs and/ or suitable timber framing to achieve strength grade requirements for framing assembly and adequately support weight of door.

-	Services penetrations: Allow for associated fire stopping.

K10/475	METAL FURRINGS FOR WALL LININGS

-	Setting out: Accurately aligned and plumb.

-	Vertical furring positions: Equal vertical centres to suit specified linings, maintaining sequence across openings. Position adjacent to angles and openings.

-	Additional vertical furrings: To support vertical edges of boards and at junctions with partitions.

-	Horizontal furring positions: To provide continuous support to edges of boards.

-	Adhesive bedding to furrings:

-	Dabs: Length 200 mm (minimum). Located at ends of furrings and thereafter at 450 mm (maximum) centres.

-	Junctions with partitions: Continuous bed with no gaps across cavity.

K10/505	INSTALLING MINERAL WOOL INSULATION

-	Fitting insulation: Closely butted joints and no gaps. Use fasteners to prevent slumping or displacement.

-	Services:

-	Electrical cables overlaid by insulation: Sized accordingly.

-	Ceilings: Cut insulation around electrical fittings, etc.

K10/510	SEALING GAPS AND AIR PATHS

-	Location of sealant: To perimeter abutments and around openings.

-	Pressurized shafts and ducts: At board-to-board and board-to-metal frame junctions.

-	Application: To clean, dry and dust free surfaces as a continuous bead with no gaps.

-	Gaps greater than 6 mm between floor and underside of plasterboard: After sealing, fill with jointing compound.

K10/555	FIRE STOPPING AT PERIMETERS OF DRY LINING SYSTEMS

-	Material: Tightly packed mineral wool or intumescent mastic/ sealant.

-	Application: To perimeter abutments to provide a complete barrier to smoke and flame.

K10/560	JOINTS BETWEEN BOARDS

-	Tapered edged plasterboards:

-	Bound edges: Lightly butted.

-	Cut/ Unbound edges: 3 mm gap.

-	Square edged plasterboards: 3 mm gap.

-	Square edged fibre reinforced gypsum boards: 5 mm gap.

K10/565	VERTICAL JOINTS

-	Joints: Centre on studs.

-	Partitions: Stagger joints on opposite sides of studs.

-	Two layer boarding: Stagger joints between layers.

K10/570	HORIZONTAL JOINTS

-	Surfaces exposed to view: Horizontal joints not permitted. Seek instructions where height of partition/ lining exceeds maximum available length of board.

-	Two layer boarding: Stagger joints between layers by at least 600 mm.

-	Edges of boards: Support using additional framing.

-	Two layer boarding: Support edges of outer layer.

K10/580	INSULATION BACKED PLASTERBOARD

-	General: Do not damage or cut away insulation to accommodate services.

-	Installation at corners: Carefully cut back insulation or plasterboard as appropriate along edges of boards to give a continuous plasterboard face, with no gaps in insulation.

K10/590	FIXING PLASTERBOARD TO METAL FRAMING/ FURRINGS

-	Partitions/ Wall linings: Fix securely and firmly at the following centres (maximum):

-	Single layer boarding: To all framing at 300 mm centres. Reduce to 200 mm centres at external angles.

-	Multi-layer boarding: Face layer at 300 mm centres, and previous layers around perimeters at 300 mm centres.

-	Ceilings: 230 mm. Reduce to 150 mm at board ends and at lining perimeters.

-	Position of screws from edges of boards (minimum): 10 mm.

-	Screw heads: Set in a depression. Do not break paper or gypsum core.

K10/610	FIXING PLASTERBOARD TO TIMBER

-	Fixing to timber: Securely at the following centres (maximum):

-	Nails: 150 mm.

-	Screws to partitions/ wall linings: 300 mm. Reduce to 200 mm at external angles.

-	Screws to ceilings: 230 mm.

-	Position of nails/ screws from edges of boards (minimum):

-	Bound edges: 10 mm.

-	Cut/ Unbound edges: 13 mm.

-	Position of nails/ screws from edges of timber supports (minimum): 6 mm.

K10/620	FIXING PLASTERBOARD WITH ADHESIVE DABS

-	Setting out boards: Accurately aligned and plumb.

-	Fixing to substrates: Securely using adhesive dabs.

-	Adhesive dab spacings for each board:

-	Horizontally: One row along top edge and one continuous dab along bottom edge.

-	Vertically: One row along each edge and thereafter at intermediate spacings to suit size of board:

Thickness (mm)	Width (mm)	Dab centres (mm)

9.5			1200		400

9.5/12.5		 900		450

12.5			1200		600

-	Adhesive dab dimensions (width x length): At least 50-75 mm x 250 mm.

-	Position of dabs from edges/ ends of boards (minimum): 25 mm.

K10/625	FIXING INSULATION BACKED PLASTERBOARD WITH ADHESIVE DABS

-	Fixing to substrates: In addition to adhesive dab fixings, secure boards with nailable plugs in locations recommended by board manufacturer.

FINISHING

K10/650	LEVEL OF DRY LINING ACROSS JOINTS

-	Sudden irregularities: Not permitted.

-	Joint deviations: Measure from faces of adjacent boards using methods and straightedges (450 mm long with feet/ pads) to BS 8212, clause 3.3.5.

-	Tapered edge joints:

Permissible deviation (maximum) across joints when measured with feet resting on boards: 3 mm.

-	External angles:

Permissible deviation (maximum) for both faces: 4 mm.

-	Internal angles:

Permissible deviation (maximum) for both faces: 5 mm.

K10/670	SEAMLESS JOINTING TO PLASTERBOARDS

-	Cut edges of boards: Lightly sand to remove paper burrs.

-	Filling and taping: Fill joints, gaps and internal angles with jointing compound and cover with continuous lengths of paper tape, fully bedded.

-	Protection of edges/ corners: Reinforce external angles, stop ends, etc. with specified edge/ angle bead.

-	Finishing: Apply jointing compound. Feather out each application beyond previous application to give a flush, smooth, seamless surface.

-	Nail/ Screw depressions: Fill with jointing compound to give a flush surface.

-	Minor imperfections: Remove by light sanding.

K10/680	SKIM COAT PLASTER FINISH

-	Plaster type: ______ .

-	Thickness: 2-3 mm.

-	Joints: Fill and tape except where coincident with metal beads.

-	Finish: Tight, matt, smooth surface with no hollows, abrupt changes of level or trowel marks.

K10/695	INSTALLING BEADS/ STOPS

-	Cutting: Neatly using mitres at return angles.

-	Fixing: Securely using longest possible lengths, plumb, square and true to line and level, ensuring full contact of wings with substrate.

-	Finishing: After joint compounds/ plasters have been applied, remove surplus material while still wet from surfaces of beads exposed to view.