K32	PANEL CUBICLES

To be read with Preliminaries/ General conditions.

K32/112	FULLY FRAMED PANEL CUBICLES FOR ALL WC’S INCLUDING DUCTWORK FOR CISTERNS, DUCT MOUNTED BASINS, SEMI-RECESSED BASINS ETC

-	Manufacturer: Armitage Venesta

-	Product reference: Genesis Resista in area G17, Quantum Resista elsewhere

-	Panels/ Doors: see drawings

-	Core material: 13mm thick with polished radiused edges.

-	Colour: To be selected.

-	Headrails: As recommended by manufacturer

-	Legs: As recommeded by manufacturer

-	Ironmongery/ Accessories: Standard.

-	Other requirements: None.

K32/130	Privacy screenS areaS G02, F25, S10 & S26 as shown

-	Manufacturer: Armitage Venesta .

-	Product reference: As cubicles clause 112 .

-	Frame/ Supports: As recommended by manufacturer.

-	Colour: To be selected .

-	Panels: As recommended by manufacturer.

-	Colour:  To be selected .

-	Accessories: none .

-	Other requirements: none .

K32/230	PROTECTION

-	Panels: Stack flat on bearers and separate by spacers where necessary to prevent damage to or from projections.

-	Completed cubicles: Keep clean and dry, and adequately protect from damage until completion.

K32/250	INSTALLATION

-	Programming: Do not install cubicles before building is weathertight, wet trades have finished their work, wall and floor finishes are complete, and the building is well dried out.

-	Accuracy: Set out to ensure frames and/ or panels and doors are plumb, level and accurately aligned.

-	Modifications: Do not cut, plane or sand prefinished components except where shown on drawings.

-	Fixing: Secure using methods and fasteners recommended by the cubicle manufacturer. Prevent pulling away, bowing or other distortions to frames, panels and doors.

-	Moisture and thermal movement: Make adequate allowance for future movement.