# Q31 External planting

To be read with Preliminaries/ General conditions.

GENERAL INFORMATION/ REQUIREMENTS

120	CLIMATIC CONDITIONS

•	 General: Carry out the work while soil and weather conditions are suitable.

-	Strong winds: Do not plant.

125	TIMES OF YEAR FOR PLANTING

•	 Deciduous trees and shrubs: Late October to late March.

•	 Conifers and evergreens: September/ October or April/ May.

•	 Herbaceous plants (including marginal): September/ October or March/ April.

•	 Container grown plants: At any time if ground and weather conditions are favourable.

-	Watering and weed control: Provide as necessary.

•	 Dried bulbs, corms and tubers: September/ October.

•	 Colchicum (crocus): July/ August.

•	 Green bulbs: After flowering in spring.

•	 Wildflower plugs: Late August to mid November or March/ April.

•	 Aquatic plants: May/ June or September/ October.

145	WATERING

•	 Quantity: Wet full depth of topsoil.

•	 Application: Even and without damaging or displacing plants or soil.

•	 Frequency: As necessary to ensure establishment and continued thriving of planting.

150	WATER RESTRICTIONS

•	 General: If water supply is or is likely to be restricted by emergency legislation, do not

carry out planting until instructed. If planting has been carried out, obtain instructions

on watering.

200	PLANTS/ TREES - GENERAL

•	 Condition: Materially undamaged, sturdy, healthy and vigorous.

•	 Appearance: Of good shape and without elongated shoots.

•	 Hardiness: Grown in a suitable environment and hardened off.

•	 Health: Free from pests, diseases, discoloration, weeds and physiological disorders.

•	 Budded or grafted plants: Bottom worked.

•	 Root system and condition: Balanced with branch system.

-	Standard: The National Plant Specification.

•	 Species: True to name.

•	 Origin/ Provenance: British grown.

- Definition: Origin and Provenance have the meaning given in the National

Plant Specification.

215	PLANTS/ TREES - SPECIFICATION CRITERIA

•	 Name, forms, dimensions, provenance and other criteria: As scheduled and defined in

the National Plant Specification.

235	CONTAINER GROWN PLANTS/ TREES

•	 Growing medium: With adequate nutrients for plants to thrive until permanently planted.

• Plants: Centred in containers, firmed and well watered.

• Root growth: Substantially filling containers, but not root bound, and in a condition

conducive to successful transplanting.

•	 Hardiness: Grown in the open for at least two months before being supplied.

•	 Containers: With holes adequate for drainage when placed on any substrate

commonly used under irrigation systems.

260	PLANT/ TREE SUBSTITUTION

•	 Plants/ trees unobtainable or known to be likely to be unobtainable at time of ordering:

Submit alternatives, stating:

-	Price.

-	Difference from specified plants/ trees.

•	 Approval: Obtain before making any substitution.

265	PLANT HANDLING STORAGE TRANSPORT AND PLANTING

•	 Standard: To HTA 'Handling and establishing landscape plants'.

•	 Frost: Protect plants from frost.

•	 Handling: Handle plants with care. Protect from mechanical damage and do not

subject to shock, e.g. by dropping from a vehicle.

•	 Plant packaging: Black polyethylene bags.

•	 Packaging of bulk quantities: not applicable.

•	 Planting: Upright or well balanced with best side to front.

290	SURPLUS MATERIAL

•	 Subsoil, stones, debris, wrapping material, canes, ties, temporary labelling, rubbish,

prunings and other arisings: Remove.

PREPARATION OF PLANTING BEDS/ PLANTING MATERIALS

361	COMPOSTNEW PLANTING BED AND TREE PIT IN NURSERY PLAYGROUND

•	 Locations: All planting areas.

•	 Type: Sanitized and stabilized compost.

•	 Manufacturer/ Supplier: Contractor's choice.

-	Product reference: Submit proposals.

•	 Standard: To PAS 100.

•	 Horticulture parameters:

-	pH (1:5 water extract): 7.0-8.7.

-	Electrical conductivity (maximum, 1:5 water extract): 200 mS/m.

-	Moisture content (m/m of fresh weight): 35-55%.

-	Organic matter content (minimum): 25%.

-	Grading (air dried samples): 99% passing 25 mm screen, and 90% passing 10

mm screen mesh aperture.

-	Carbon:Nitrogen ration (maximum): 20:1.

•	 Texture: Friable.

•	 Objectionable odour: None.

•	 Composting Association certification: Required.

•	 Application: Spread evenly.

-	Timing: Apply prior to cultivation.

-	Rate: As plant nursery recommendations.

•	 Other requirements: None.

375	CULTIVATION

•	 Compacted topsoil: Break up to full depth.

•	 Cultivation: Loosen, aerate and break up soil into particles of 2-8 mm.

-	Depth: 250 mm.

-	Timing: Within a few days before planting.

-	Weather and ground conditions: Suitably dry.

•	 Surface: Leave regular and even.

•	 Levels: to suit.

•	 Undesirable material brought to the surface: Remove visible weeds, roots and large

stones with any dimension exceeding 50 mm.

•	 Soil within root spread of trees and shrubs to be retained: Do not dig or cultivate.

PLANTING SHRUBS/ HERBACEOUS PLANTS/ BULBS

405	SHRUB PLANTING PITS

•	 Timing: Excavate 1-2 days (maximum) before planting.

•	 Sizes: Wide enough to accommodate roots when fully spread and 75 mm deeper than

root system.

•	 Pit bottom improvement Break up to a depth of 150 mm, incorporating 10 kg compost.

•	 Backfilling material: Reuse excavated material..

420	CLIMBING PLANTS

•	 Planting: 150 mm clear of supporting structure (e.g. wall/ fence) with roots spread

outward.

-	Branches: Lightly securedd to supports.

•	 Climber supports: Softwood and Stainless steel wire.

-	Base height: As detail drawing.

-	Extent: As detail drawing.

-	Centres: As detail drawing.

-	Distance from wall: As detail drawing.

•	 Fixings: Galvanized screw eyes.

-	Centres: 1 m.

480	AFTER PLANTING

•	 Watering: Immediately after planting, thoroughly and without damaging or displacing

plants or soil.

•	 Firming: Lightly firm soil around plants and fork and/ or rake soil, without damaging

roots, to a fine tilth with gentle cambers and no hollows.

•	 Top dressing: Sanitized and stabilized compost.

-	Depth: 25 mm.

485	MULCHING PLANTING BEDS

•	 Material: Play grade bark mulch.

-	Purity: Free of pests, disease, fungus and weeds.

-	Recycled content: Contractor's choice.

•	 Preparation: Clear all weeds. Water soil thoroughly.

•	 Coverage: 1 m³er 10 m²

•	 Finished level of mulch: to suit.

590	MULCHING TREES

•	 Material: Play grade bark mulch.

-	Purity: Free of pests, disease, fungus and weeds.

-	Recycled content: Contractor's choice.

•	 Preparation: Clear all weeds. Water soil thoroughly.

•	 Coverage: 50 litres per tree position, 75 mm depth.

•	 Finished level of mulch: to suit.

PROTECTING/ MAINTAINING/ MAKING GOOD DEFECTS

720	FAILURES OF PLANTING

•	 Defects due to materials or workmanship not in accordance with the Contract: Plants/

trees/ shrubs that have failed to thrive.

-	Exclusions: Theft or malicious damage after completion.

-	Rectification: Replace with equivalent plants/ trees/ shrubs.

•	 Replacements: To match size of adjacent or nearby plants of same species or match

original specification, whichever is the greater.

•	 Timing of making good: In accordance with an agreed defects rectification programme .

740	CLEANLINESS

•	 Soil and arisings: Remove from hard sufaces and grassed areas.

•	 General: Leave the works in a clean tidy condition at completionn and after any

maintenance operations.