P21	IRONMONGERY

To be read with Preliminaries/ General conditions.

P21/010	QUANTITIES AND LOCATIONS

-	Quantities and locations of ironmongery are to be based on ironmongery schedules

-	Fixing: As sections L10 and L20.

GENERALLY

P21/120	IRONMONGERY RANGE TO MATCH THAT USED IN FIRST PHASE WORKS

-	Source: Single co-ordinated range.

-	Notification: Submit details of selected range, manufacturer and/ or supplier.

-	Principal material/ finish: satin anodised aluminium .

-	Items unavailable within selected range: Submit proposals.

P21/140	SAMPLES

-	General: Before placing orders with suppliers submit labelled samples of the following: lever handles, pull handles, thumb turns, bathroom lock and emergency release, door stops .

-	Conformity: Retain samples on site for the duration of the contract. Ensure conformity of ironmongery as delivered with labelled samples.

P21/170	IRONMONGERY FOR FIRE DOORS

-	Relevant products: Ironmongery fixed to, or morticed into, the component parts of a fire resisting door assembly.

-	Compliance: Ironmongery included in successful tests to BS 476-22 or BS EN 1634-1 on door assemblies similar to those proposed.

-	Certification: Submit CERTIFIRE certificates .

-	Melting point of components (except decorative non functional parts): 800°C minimum.

P21/180	CATEGORY OF DUTY FOR DOOR IRONMONGERY

-	Standard: To DD 171.

-	Category of duty of doors: Medium duty .

-	General: Durability of ironmongery components to be compatible with stated category of duty of each door leaf.

-	Exclusions: Ironmongery with specific duty or ‘category of use’ defined elsewhere.

-	Documentation: Before placing orders with suppliers submit documentation showing product compliance with stated category of duty.

DOOR HANGING DEVICES

P21/310	SINGLE AXIS DOOR HINGES

-	Standard: To BS EN 1935.

-	Hinges to doors on escape routes and fire/ smoke control doors: CE marked.

-	Manufacturer: As clause 120.

-	Material/ finish: Grade 1.4401 (316) satin stainless steel .

-	Hinge grade: Not less than 11 .

DOOR OPERATING DEVICES

P21/410	OVERHEAD DOOR CLOSERS AS IRONMONGERY SCHEDULE

-	Standard: To BS EN 1154.

-	Door closing devices to fire/ smoke control doors: CE marked.

-	Manufacturer: Contractor’s choice .

-	Type: Face fixed .

-	Power size: Minimum 3 .

-	Casing finish: As door furniture .

-	Operational adjustment:

-	Variable power: Matched to size, weight and location of doors.

-	Latched doors: Override latches and/ or door seals when fitted.

-	Unlatched doors: Hold shut under normal working conditions.

-	Closing against smoke seals of fire doors: Positive. No gaps.

P21/471	ELECTROMAGNETIC HOLD OPEN/ SWING-FREE DEVICES AS IRONMONGERY SCHEDULE

-	Standard: To BS EN 1155.

-	Electromagnetic devices to fire/ smoke control doors: CE marked.

-	Manufacturer: Contractors choice

-	Type: Face fixed .

-	Electric supply: To be advised .

-	Means of release: Alarm system and/ or failure of power supply.

-	Test switch: Located in a convenient position adjacent to door.

-	Operational adjustment of integral closer:

-	Variable power: Matched to size, weight and location of doors.

-	Latched doors: Override latches and/ or door seals when fitted.

-	Unlatched doors: Hold shut under normal working conditions.

DOOR SECURING DEVICES

P21/510	THIEF RESISTANT DOOR LOCKS TO ALL DOORS

-	Standard: To BS 3621 and Kitemarked.

-	Manufacturer: As clause 120 .

-	Type: To match original doors throughout the school

-	Backset: To suit doors.

-	Material/ finish: As door furniture .

-	Keying: In master keyed suite.

P21/540	DOOR LATCHES TO ALL DOORS

-	Standard: To BS EN 12209.

-	Manufacturer: As clause 120 .

-	Type: As schedule .

-	Backset: To suit doors .

-	Material/ finish: Faceplate as door furniture.

-	Latch spring strength: Select to prevent unsprung lever handles drooping.

DOOR FURNITURE

P21/610	LEVER HANDLES AS SCHEDULE

-	Standard: To BS EN 1906.

-	Manufacturer: As clause 120  .

-	Style: Round pattern with return to door .

-	Size: 20mm diameter .

-	Material/ finish: Satin anodised aluminium .

-	Mounting: As schedule .

P21/641	PULL HANDLES AS SCHEDULE

-	Standard: To BS 8424.

-	Manufacturer: As clause 120 .

-	Shape: round pattern with return to door .

-	Diameter: 20mm diameter .

-	Distance between centres: As scheduled .

-	Material/ finish: Satin anodised aluminium

-	Mounting: As schedule.

P21/690	KICK PLATES AS SCHEDULE

-	Manufacturer: As clause 120  .

-	Material/ finish: Satin anodised aluminium .

-	Additional requirements: To be advised .

P21/710	ESCUTCHEONS AS SCHEDULE

-	Manufacturer: As clause 120  .

-	Material/ finish: Satin anodised aluminium

-	Keyhole type: As schedule .

P21/720	DOOR STOPS

-	Manufacturer: As clause 120

-	Type: Wall mounted face fixed SAA finish with projection to suit door. .

-	Usage: To all doors opening against walls .

P21/760	DOOR HOLDERS AS SCHEDULE

-	Manufacturer: As clause 120

-	Type: Cabin hooks .

-	Size: 150mm (to be confirmed) .

-	Material/ finish: Satin anodised aluminium .

P21/811	DOOR MOUNTED COAT HOOKS AS SCHEDULE

-	Manufacturer: As clause 120

-	Type: double .

-	Material/ finish: Satin anodised aluminium .

P21/850	THRESHOLD WEATHERSTRIP – SEE DETAILS

-	Manufacturer: Sealmaster or equal and approved .

-	Product reference: BSD for outward opening doors .

-	Type: See details.

-	Material/ finish: Mill finish aluminium .

P21/855	WEATHERSTRIP TO EXTERNAL DOORS, HEAD AND JAMBS.

-	Manufacturer: Sealmaster or equal and approved ..

-	Type: Neoprene in metal carrier

-	Material/ finish: Mill finish aluminium .