Z20	FIXINGS / ADHESIVES

To be read with Preliminaries/ General conditions.

Z20/110	FIXINGS GENERALLY

-	Integrity of supported components: Types, sizes and quantities of fasteners/ packings and spacings of fixings selected to retain supported components without distortion or loss of support.

-	Components/ substrates/ fasteners of dissimilar metals: Fixed with isolating washers/ sleeves to avoid bimetallic corrosion.

-	General usage: To recommendations of fastener manufacturers and/ or manufacturers of components, products or materials fixed and fixed to.

-	Appearance: As approved samples.

Z20/130	FASTENER DURABILITY

-	Fasteners in external construction:

-	Fasteners not directly exposed to weather: Of corrosion resistant material or with a corrosion resistant finish.

-	Fasteners directly exposed to weather: Of corrosion resistant material.

Z20/140	FIXINGS THROUGH FINISHES

-	Penetration of fasteners/ plugs into substrate: To achieve a secure fixing.

Z20/150	PACKINGS

-	Function: To take up tolerances and prevent distortion of materials/ components.

-	Materials: Noncompressible, noncorrodible, rot proof.

-	Locations: Not within zones to be filled with sealant.

Z20/160	CRAMP FIXINGS

-	Cramp positions: Maximum 150 mm from each end of frame sections and at 600 mm maximum centres.

-	Fasteners: Cramps fixed to frames with screws of same material as cramps.

-	Cramp fixings in masonry work: Fully bedded in mortar.

Z20/230	PELLETED COUNTERSUNK FIXINGS

-	Finished level of countersunk screw heads: Minimum 6 mm below timber surface.

-	Pellets: Cut from matching timber, grain matched and glued in to full depth of hole.

-	Finished level of pellets: Flush with surface.

Z20/250	POWDER ACTUATED FIXING SYSTEMS

-	Powder actuated fixing tools: To BS 4078-2 and Kitemark certified. Operatives trained and certified as competent by tool manufacturer.

-	Types of fastener, accessories and consumables: As recommended by tool manufacturer.

-	Protective coating to exposed fasteners used externally or in other locations subject to dampness: Zinc rich primer to fastener heads.

Z20/510	ADHESIVES

-	Storage/ Usage: In accordance with manufacturer’s and statutory requirements.

-	Surfaces: Clean. Regularity and texture adjusted to suit bonding and gap filling characteristics of adhesive.

-	Finished adhesive joints: Fully bonded. Free of surplus adhesive.