N10	GENERAL FIXTURES/ FURNISHINGS/ EQUIPMENT

To be read with Preliminaries/ General conditions.

PRODUCTS

N10/110	PURPOSE MADE DESKS AREA G07 AS DRAWINGS

-	Supports:

-	Material: 18mm mdf/ply on ex 25 x 50mm sw battens/framing.  Circular grey metal legs from Hafele or equal approved.

-	Worktops:

-	Material: Desktop linoleum by Forbo Nairn – colour to be selected.

-	Edge: 2 x 22mm unlipped birch faced ply, with varnish finish.

N10/110/A	PURPOSE MADE RECEPTION DESK AREA G14 AS DRAWINGS

-	Supports:

-	Material: 22mm Douglas Fir faced ply to vertical surfaces, outer faces clad with 19mm Douglas Fir boarding to match elsewhere.  SW battens/framing to suit.  18mm mdf/ply worktop base on sw battens/framing, ex 50mm Douglas Fir brackets, and circular grey metal legs from Hafele or equal approved.

-	Worktops:

-	Material: Desktop linoleum by Forbo Nairn – colour to be selected.

-	Edge: Ex 100 x 50 Douglas Fir lipping.

-	Other requirements: Douglas Fir plinth/skirting, 22mm Douglas Fir faced ply lockable drawers on Hafele or equal approved drawer slides.  Trunking for electrics and black plastic cable outlets..

N10/110/B	PURPOSE MADE STORAGE UNIT AREA G07 AS DRAWINGS

-	Timber: To BS EN 942.

-	Species: unlipped 18mm birch faced ply .

-	Appearance class: J2 .

-	Moisture content on delivery: 6-10% .

-	Finishes: As section M60 .

-	Adhesive: To BS EN 301 type II .

-	Joinery workmanship: As section Z10.

-	Metalwork materials and workmanship: As section Z11.

-	Other requirements: Hinges & adjustable shelves as drawings.

N10/110/C	PURPOSE MADE STORAGE UNIT AREA G18 AS DRAWINGS

-	Timber: To BS EN 942.

-	Species: unlipped 18mm birch faced ply, except 25mm for worktop and for doors .

-	Appearance class: J2 .

-	Moisture content on delivery: 6-10% .

-	Finishes: As section M60 .

-	Adhesive: To BS EN 301 type II .

-	Joinery workmanship: As section Z10.

-	Metalwork materials and workmanship: As section Z11.

-	Other requirements: Adjustable shelves as drawings.  Hafele top hung HAWA-Dorado Infront track system, comprising 4 No 2m tracks (ref 940.40.200), 2No 2 door fitting sets (ref 406.70.120) and 2No lower guide profiles (ref 406.73.103).

N10/110/D	PURPOSE MADE STORAGE UNIT AREA G19 AS DRAWINGS

-	Timber: To BS EN 942.

-	Species: unlipped 18mm birch faced ply.

-	Appearance class: J2 .

-	Moisture content on delivery: 6-10% .

-	Finishes: As section M60 .

-	Adhesive: To BS EN 301 type II .

-	Joinery workmanship: As section Z10.

-	Metalwork materials and workmanship: As section Z11.

-	Other requirements: Hinges & adjustable shelves as drawings

N10/110/E	PURPOSE MADE STORAGE/WET AREA CUPBOARDS AND SHELVING IN CLASSROOMS, UNIT ROOMS & 1NO GROUP ROOM, AREAS G20, G25, S01, S12, S18, S21 (NOTE REDUCED SIZE) AND S28, AS DRAWINGS

-	Timber: To BS EN 942.

-	Species: unlipped 18mm birch faced ply .

-	Appearance class: J2 .

-	Moisture content on delivery: 6-10% .

-	Finishes: As section M60 .

-	Adhesive: To BS EN 301 type II .

-	Joinery workmanship: As section Z10.

-	Metalwork materials and workmanship: As section Z11.

-	Other requirements: Hinges as drawings, worktops as N10/200.

N10/110/F	PURPOSE MADE COAT LOCKERS AREAS S03 AND S14

-	Timber: To BS EN 942.

-	Species: unlipped 18mm birch faced ply.

-	Appearance class: J2 .

-	Moisture content on delivery: 6-10% .

-	Finishes: As section M60 .

-	Adhesive: To BS EN 301 type II .

-	Joinery workmanship: As section Z10.

-	Metalwork materials and workmanship: As section Z11.

-	Other requirements: Hinges & double SAA coat hooks as drawings

N10/110G	PURPOSE MADE SHELVES IN CUPBORAD AREA G05

-	Supports:

-	Material: 19 x 44 sw battens/

-	Shelf:

Unlipped 18mm birch faced ply, 6No. 300 x 850mm.

-	Finishes: As section M60 .

-	Joinery workmanship: As section Z10.

N10/163	COAT HOOKS FOR ALL CLASSROOMS AND UNIT ROOMS

- Material: 120 x 18mm mdf/sw board with double SAA coat hooks as follows:

- 20 No. hooks in area G05

- 20 No. hooks in area G11

- 40 No. hooks in area G18

- 12 No. hooks in area G25

- 12 No. hooks in area S18

- 12 No. hooks in area S28

N10/188	TOILET ROLL HOLDERS FOR EACH NEW WC

-	Manufacturer: Armitage Venesta .

-	Product reference: Jumbo toilet roll holder, ref 0302003 .

-	Finish/ Colour: Stainless steel .

N10/188/A	SOAP DISPENSERS FOR EACH WASH HAND BASIN

-	Manufacturer: Armitage Venesta .

-	Product reference: High quality soap dispenser ref 0302018 .

-	Finish: Stainless steel.

N10/188/B	HAND DRIERS, INo IN EACH NEW TOILET AREA, AS SHOWN

-	Manufacturer: Armitage Venesta .

-	Product reference: Automatic warm air dryer, ref 0302019.

-	Finish: Stainless steel .

N10/188/C	PAPER TOWEL DISPENSER, INo IN AREAS G09, G15, F07, F26, S07 & S27A

-	Manufacturer: Armitage Venesta .

-	Product reference: Paper towel dispenser, ref 0302020.

-	Finish: Stainless steel .

N10/200	WORKTOPS FOR STORAGE/WET AREAS IN ALL CLASSROOMS AND UNIT ROOMS, AS DRAWINGS

-	Classification: To BS 6222-3, Type 2 .

-	Material: Plastic laminate with matching edging.

-	Finish/ Colour: to be selected .

-	Dimensions: 600 wide .

-	Support: units as N10/110E.

N10/240	BLACKOUT BLINDS TO ALL WINDOWS IN AREA F11

-	Manufacturer: Levolux Ltd or equal approved .

- Type: 760L system roller blinds on 45mm barrel with 20mm diameter SAA bottom bar, endless stainless steel side chain and chain retaining pulley.

- Material: PK2 range flame retardant blackout fabric (colour to be selected)

.	-	Operation: Manual.

N10/240A	BLINDS TO ALL OTHER WINDOWS IN AREAS G05, G06, G07, G11, G18, G19, G25, F06, F08, F09, F10, F16, F19, F20, S01, S02, S06, S08, S11, S12, S18, S21, S23, S28

-		Manufacturer: Levolux Ltd or equal approved .

- Type: 760L system roller blinds on 45mm barrel with 20mm diameter SAA bottom bar, endless stainless steel side chain and chain retaining pulley.

- Material: SV1% range flame retardant open weave Levolu screen fabric (colour to be selected)

.	-	Operation: Manual.

N10/270	MIRRORS TO ALL WC’S AS SHOWN ON DRAWINGS

-	Material: Safety glass to BS 6206, class C .

-	Quality: Free from tarnishing, discoloration, scratches and other defects visible in the designed viewing conditions. Reflection undistorted.

-	Size: see drawings .

-	Edges: square.

-	Fixing: CP dome topped screws with polyethylene sleeves and washers.

-	Installation: Accurately with sides vertical.

N10/300	ENTRANCE MATTING AND FRAME AREAS G14, & G20, SIZES AS SHOWN ON DRAWINGS

-	Base: Screed .

-	Manufacturer: Jaymart Rubber and Plastics Ltd .

Product reference: Street-King entrance matting, with closed construction, and with Well-Maid aluminium matwell frame.

N10/300/A	ENTRANCE MATTING AREAS G05, G11 & G25 SIZES AS SHOWN ON DRAWINGS

-	Base: Screed .

Manufacturer and reference: Bonar Floors Ltd, High Holborn Road, Ripley, Derbyshire DE5 3NT. Tel: 01773 744121. Fax: 01773 744142.

Web: www.bonarfloors.com

Reference: Coral Classic FR

Colour: to be selected

Sizes: as shown on drawings

N10/350	DRINKING FOUNTAINS IN ALL CLASSROOMS AND UNIT ROOMS, INCLUDING FIRST AND SECOND FLOOR REAR CLASSROOMS AS SHOWN ON DRAWINGS

-		Ideal Standard Purita stainless steel fountain ref S5435 (53400Z5)

N10/350A	GRAB RAIL AREAS G02, F25, S10 & S26

-	Item: Vertical grab rail adjacent to urinals.

-	Manufacturer: contractor’s choice.

-	Size 750mm high.

-	Finish/ Colour: white .

N10/350B	TRIM AT BASE OF COLUMN AREA G20, AS SHOWN ON DRAWINGS

-	Manufacturer: Contractor’s choice .

-	Size and shape: as shown.

-	Finish: galvanised steel .

-	Other requirements: 20mm insulation to face of timber as shown.

EXECUTION

N10/710	MOISTURE CONTENT OF WOOD AND WOOD BASED BOARDS

-	Temperature and humidity: During delivery, storage, fixing and to handover maintain conditions to suit specified moisture contents of timber components.

-	Testing: When instructed, test components with approved moisture meter to manufacturer's recommendations.

N10/720	INSTALLATION GENERALLY

-	Fixing and fasteners: As section Z20.

N10/760	SEALANT POINTING

-	Material: Silicone based to BS 5889, Type B with fungicide.

-	Colour: to  be agreed .

-	Manufacturer: Contractor’s choice ..

-	Application: As section Z22.

N10/770	TRIMS

-	Lengths: Wherever possible, unjointed between angles or ends of runs.

-	Running joints: Where unavoidable, obtain approval of location and method of jointing.

-	Angle joints: Mitred.

COMPLETION

N10/910	GENERAL

-	Doors and drawers: Accurately aligned, not binding. Adjusted to ensure smooth operation.

-	Ironmongery: Checked, adjusted and lubricated to ensure correct functioning.

N10/920	APPLIANCES

-	Test: Run for a full cycle, ensure that all functions and features work correctly.

-	Documentation: Submit guarantees, instruction manuals etc.